DC

Practical No 5

clc;

clear all;

close all;

M=input('Please enter the number of message symbols=');

SNR=input('Please enter the SNR in dB=');

x=randi(M-1,1000,1);

y=pskmod(x,M);

yn=awgn(y,SNR, 'measured');

scatterplot(y);

scatterplot(yn);

xr=pskdemod(yn,M);

[num_err, err_rate]=symerr(x,xr)

Command Window

Please enter the number of message symbols=16

Please enter the SNR in dB=49

Practical 6

clc;

clear all;

close all;

M=input('please enter the number of message symbols=');

SNR=input('please enter the SNR in dB=');

x = randi([0 M-1], 1000, 1);

y=qammod(x,M);

scatterplot(y);

yn=awgn(y,SNR, 'measured');

scatterplot(yn);

xr=qamdemod(yn,M);

[num_err,err_rate]=symerr(x,xr)

Command window

please enter the number of message symbols=16

please enter the SNR in dB=40

Practical No 7

clc;

clear all;

close all;

nr_data_bits=9100;

b_data=(randn(1,nr_data_bits))>0;

b=[b_data];

d=zeros(1,length(b));

%generation of bpsk

for n=1:length(b)

if (b(n)==0)

d(n)=exp(j*2*pi);

end

if (b(n)==1)

d(n)=exp(j*pi);

end

end

disp(d)

bpsk=d;

%plotting of bpsk data

figure(1);

plot(d,'o');

axis([-2 2 -2 2]);

grid on;

xlabel('Real');

ylabel('Imaginary');

title('BPSK constellation');

%addition of noise

SNR=0:30;

BER1=[];

SNR1=[];

for SNR=0:length(SNR);

sigma=sqrt(10.0^(-SNR/10.0));

snbpsk=(real(bpsk)+sigma.*randn(size(bpsk)))+i.*(imag(bpsk)+sigma.*randn(size(bpsk)))

;

%plotting bpsk with noise

figure(2);

plot(snbpsk,'o');

axis([-2 2 -2 2]);

grid on;

xlabel('Real');

ylabel('Imaginary');

title('BPSK constellation with noise');

%recovering of data receiver

r=snbpsk;

bhat=[real(r)<0];

bhat1=bhat;

ne=sum(b~=bhat1);

BER=ne/nr_data_bits;

BER1=[BER1 BER];

SNR1=[SNR1 SNR];

end

%plotting of BER for BPSK

figure(3);

semilogy(SNR1,BER1,'-*');

grid on;

xlabel('signal to noise ratio');

ylabel('bit error rate');

title('simulation of BER for BPSK');

legend('BER_simulated');

Practical No 8

clc;

clear all;

close all;

code_length = 0;

x = input('Enter number of symbols: ');

for m = 1:x

symbols(m) = input('Enter the symbol number: ');

p(m) = input('Enter the probability: ');

end

Hx = 0;

avglen = 0;

for m = 1:x

dict = huffmandict(symbols, p);

hcode = huffmanenco(m, dict);

dsig = huffmandeco(hcode, dict);

code_length = length(hcode);

Hx = Hx + (p(m) * (-log(p(m))) / log(2));

avglen = avglen + (p(m) * code_length);

end

display(Hx);

display(avglen);

Efficiency = (Hx / avglen) * 100;

Command Window

Enter number of symbols: 4

Enter the symbol number: 1

Enter the probability: 0.4

Enter the symbol number: 2

Enter the probability: 0.3

Enter the symbol number: 3

Enter the probability: 0.2

Enter the symbol number: 4

Enter the probability: 0.1

Practical No 9