DC

Practical No 5

clc;

clear all;

close all;

M=input('Please enter the number of message symbols=');

SNR=input('Please enter the SNR in dB=');

x=randi(M-1,1000,1);

y=pskmod(x,M);

yn=awgn(y,SNR, 'measured');

scatterplot(y);

scatterplot(yn);

xr=pskdemod(yn,M);

[num_err, err_rate]=symerr(x,xr)

Command Window

Please enter the number of message symbols=16

Please enter the SNR in dB=49

Practical 6

clc;

clear all;

close all;

M=input('please enter the number of message symbols=');

SNR=input('please enter the SNR in dB=');

x = randi([0 M-1], 1000, 1);

y=qammod(x,M);

scatterplot(y);

yn=awgn(y,SNR, 'measured');

scatterplot(yn);

xr=qamdemod(yn,M);

[num_err,err_rate]=symerr(x,xr)

Command window

please enter the number of message symbols=16

please enter the SNR in dB=40

Practical No 7

clc;

clear all;

close all;

nr_data_bits=9100;

b_data=(randn(1,nr_data_bits))>0;

b=[b_data];

d=zeros(1,length(b));

%generation of bpsk

for n=1:length(b)

if (b(n)==0)

d(n)=exp(j*2*pi);

end

if (b(n)==1)

d(n)=exp(j*pi);

end

end

disp(d)

bpsk=d;

%plotting of bpsk data

figure(1);

plot(d,'o');

axis([-2 2 -2 2]);

grid on;

xlabel('Real');

ylabel('Imaginary');

title('BPSK constellation');

%addition of noise

SNR=0:30;

BER1=[];

SNR1=[];

for SNR=0:length(SNR);

sigma=sqrt(10.0^(-SNR/10.0));

snbpsk=(real(bpsk)+sigma.*randn(size(bpsk)))+i.*(imag(bpsk)+sigma.*randn(size(bpsk)))

;

%plotting bpsk with noise

figure(2);

plot(snbpsk,'o');

axis([-2 2 -2 2]);

grid on;

xlabel('Real');

ylabel('Imaginary');

title('BPSK constellation with noise');

%recovering of data receiver

r=snbpsk;

bhat=[real(r)<0];

bhat1=bhat;

ne=sum(b~=bhat1);

BER=ne/nr_data_bits;

BER1=[BER1 BER];

SNR1=[SNR1 SNR];

end

%plotting of BER for BPSK

figure(3);

semilogy(SNR1,BER1,'-*');

grid on;

xlabel('signal to noise ratio');

ylabel('bit error rate');

title('simulation of BER for BPSK');

legend('BER_simulated');

Practical No 8

clc;

clear all;

close all;

x = input('Enter number of symbols: ');

symbols = zeros(1, x);

p = zeros(1, x);

for m = 1:x

symbols(m) = input(['Enter the symbol number ', num2str(m), ': ']);

p(m) = input(['Enter the probability of symbol ', num2str(m), ': ']);

end

dict = huffmandict(symbols, p);

disp('Huffman Dictionary:');

disp(dict);

Hx = 0;

for m = 1:x

Hx = Hx + p(m) * (-log2(p(m)));

end

avglen = 0;

for m = 1:x

codeword = dict{m, 2};

avglen = avglen + p(m) * length(codeword);

end

Efficiency = (Hx / avglen) * 100;

fprintf('\nEntropy H(X) = %.4f bits/symbol\n', Hx);

fprintf('Average Codeword Length = %.4f bits/symbol\n', avglen);

fprintf('Coding Efficiency = %.2f%%\n', Efficiency);

Command Window

Enter number of symbols: 4

Enter the symbol number: 1

Enter the probability: 0.4

Enter the symbol number: 2

Enter the probability: 0.3

Enter the symbol number: 3

Enter the probability: 0.2

Enter the symbol number: 4

Enter the probability: 0.1

Practical No 9

clc; clear all;close all;

Px=input('Enter the input probability matrix=');

CP=input('Enter the conditional probability matrix=');

s=size(CP)

%calculate the joint probability matrixJP=0;

for i=1:s(1);

for j=1:s(1);

JP(i,j)=Px(i)*CP(i,j);end

end

%to find o/p matrix

for j=1:s(2)

Py(j)=sum(JP(:,j));

end

%to find entropy`

Hx=0;

for i=1:s(1)

Hx=Hx+Px(i)*log2(1/Px(i));end

display('The source entropy is =');Hx

%to find o/p entropy

Hy=0;

for j=1:s(2)

Hy=Hy+Py(j)*log2(1/Py(j));end

display('The o/p entropy is =');Hy

%to find joint probability

Hxy=0;

for i=1:s(1);

for j=1:s(2);

Hxy=Hxy+JP(i,j)*log2(1/JP(i,j));end

end

display('The joint prabability is =');Hxy

%to find conditional entropy(H(x/y)

cond_Hxy=Hxy-Hy

%to find conditional entropy(H(y/x)

cond_Hxy=Hxy-Hx

%mutual information

Ixy=Hx+Hy-Hxy

Output

Enter the input probability matrix=[1/3;2/3]

Enter the conditional probability matrix=[2/3  1/3;1/3  2/3]

Practical 10

clc;

clear all;

k=input('enter the length of message word:');

n=input('enter the length of codeword:');

P=input('enter the parity matrix:');

G= [eye(k) P]

m=input('enter the message word: ');

C= encode(m,n,k, 'linear',G) % codeword

H= [P' eye(n-k)] % parity check matrix

dtable = syndtable(H) % Syndrome decoding table

R=input('enter the received codeword');

S= R*H'

S= rem(S,2) % Syndrome vector

S_D = bi2de (S, 'left-msb')

if(S_D==0)

disp('The received codeword is valid');

else

disp('The received codeword is invalid');

E = dtable (S_D+1,:) % error pattern

disp('The corrected codeword is:');

CC = rem(R+E,2) % MODULO-2 addition of Rand E

end

output

enter the length of message word:3

enter the length of codeword:6

enter the parity matrix:[ 1 1 1 ; 1 1 0 ; 0 1 1 ]

G =

1     0     0     1     1     1

0     1     0     1     1     0

0     0     1     0     1     1

enter the message word:  [ 0 1 1 ]

C =

0     1     1     1     0     1

H =

1     1     0     1     0     0

1     1     1     0     1     0

1     0     1     0     0     1

Single-error patterns loaded in decoding table.  1 rows remaining.

2-error patterns loaded.  0 rows remaining.

dtable =

0     0     0     0     0     0

0     0     0     0     0     1

0     0     0     0     1     0

0     0     1     0     0     0

0     0     0     1     0     0

1     0     0     0     1     0

0     1     0     0     0     0

1     0     0     0     0     0

enter the received codeword [ 1 0 1 1 0 1 ]

S =

2     2     3

S =

0     0     1

S_D =

1

The received codeword is invalid

E =

0     0     0     0     0     1

The corrected codeword is:

CC =

1     0     1     1     0     0

>>